# Sign Language Capture and Illustration System

Project Code – Separate Files

## 1. collect_data.py

Python program for collecting flex-sensor data through Bluetooth and saving it as CSV data.

import serial
import csv
import os
import time

BT_PORT = "COM5"
BAUD_RATE = 9600
SERIAL_TIMEOUT = 3

DATA_FILE = "ml/data/gesture_data.csv"

FEATURES = ["thumb", "index", "middle", "ring", "pinky"]

GESTURES = [
    "Hello", "Yes", "No", "ThankYou", "Please",
    "Sorry", "Help", "Stop", "Good", "Bad",
    "Come", "A", "B", "C", "D", "L", "V", "S", "F"
]

SAMPLES_PER_GESTURE = 60


def connect_bluetooth():
    try:
        ser = serial.Serial(
            BT_PORT,
            BAUD_RATE,
            timeout=SERIAL_TIMEOUT
        )
        time.sleep(2)
        print("Bluetooth connected successfully.")
        return ser

    except Exception as e:
        print("Bluetooth connection failed:", e)
        return None


def collect_data():
    ser = connect_bluetooth()

    if ser is None:
        return

    os.makedirs(os.path.dirname(DATA_FILE), exist_ok=True)

    with open(DATA_FILE, "a", newline="") as file:
        writer = csv.writer(file)

        if os.path.getsize(DATA_FILE) == 0:
            writer.writerow(FEATURES + ["gesture"])

        for gesture in GESTURES:

            print("\nGet ready for gesture:", gesture)
            input("Press ENTER to start collecting...")

            count = 0

            while count < SAMPLES_PER_GESTURE:

                try:
                    line = ser.readline().decode(
                        "utf-8",
                        errors="ignore"
                    ).strip()

                    values = line.split(",")

                    if len(values) != 5:
                        continue

                    sensor_values = []

                    for value in values:
                        adc_value = int(value)

                        scaled_value = int(
                            adc_value * 4095 / 1023
                        )

                        sensor_values.append(scaled_value)

                    writer.writerow(
                        sensor_values + [gesture]
                    )

                    count += 1

                    print(
                        f"{gesture}: "
                        f"{count}/{SAMPLES_PER_GESTURE}"
                    )

                except Exception:
                    continue

    ser.close()

    print("\nData collection completed.")
    print("Data saved in:", DATA_FILE)


if __name__ == "__main__":
    collect_data()

## 2. glove_sender.ino

Arduino program for reading five flex sensors and sending the sensor values through an HC-05 Bluetooth module.

#include <SoftwareSerial.h>

const int flexPins[5] = {
  A0, A1, A2, A3, A4
};

const int BT_RX = 2;
const int BT_TX = 3;

SoftwareSerial bluetooth(BT_RX, BT_TX);

const unsigned long SEND_INTERVAL = 50;

unsigned long lastSendTime = 0;

const int FILTER_SIZE = 4;

int sensorHistory[5][FILTER_SIZE];
int historyIndex = 0;


int getSmoothedValue(int sensorIndex, int newValue)
{
  sensorHistory[sensorIndex][historyIndex] = newValue;

  long sum = 0;

  for (int i = 0; i < FILTER_SIZE; i++)
  {
    sum += sensorHistory[sensorIndex][i];
  }

  return sum / FILTER_SIZE;
}


void setup()
{
  Serial.begin(9600);
  bluetooth.begin(9600);

  for (int sensor = 0; sensor < 5; sensor++)
  {
    for (int i = 0; i < FILTER_SIZE; i++)
    {
      sensorHistory[sensor][i] = 0;
    }
  }

  Serial.println("Sign Language Glove Started");
  bluetooth.println("Sign Language Glove Started");
}


void loop()
{
  unsigned long currentTime = millis();

  if (currentTime - lastSendTime >= SEND_INTERVAL)
  {
    lastSendTime = currentTime;

    int sensorValues[5];

    for (int i = 0; i < 5; i++)
    {
      int rawValue = analogRead(flexPins[i]);

      sensorValues[i] =
        getSmoothedValue(i, rawValue);
    }

    historyIndex++;

    if (historyIndex >= FILTER_SIZE)
    {
      historyIndex = 0;
    }

    for (int i = 0; i < 5; i++)
    {
      bluetooth.print(sensorValues[i]);

      if (i < 4)
      {
        bluetooth.print(",");
      }
    }

    bluetooth.println();

    for (int i = 0; i < 5; i++)
    {
      Serial.print(sensorValues[i]);

      if (i < 4)
      {
        Serial.print(",");
      }
    }

    Serial.println();
  }
}